0.56inch DC 0-100V Three Wire LCD Blue Light Digital Voltmeter Red

- Product Features:

- This voltmeter is compact, range DC DC0-100V (two decimal places)

- Need additional auxiliary power supply, power supply voltage range is ultra-wide, up to 3.5-28V, can work in 5V, 9V, 12V, 24V, etc., no need to stabilize the voltage; power supply has a reverse connection protection, reverse not burned

- Displays are using high brightness LED digital tube, the display effect is super cool.

- Is manufactured by fully automatic SMD process, with exquisite craftsmanship.

- Note: This kind of electrician meter needs to be installed by the professional who knows electrician and electronic, please be careful to buy it for the ordinary consumers who do not know how to install and use it!

- Technical Parameters

- Test range: DC0-100V (wide range), note: the maximum input voltage can not exceed 100V, otherwise there is a risk of burnout

- TPower supply voltage: 3.5-28V

- T Minimum resolution: 0.1V

- Display mode: three 0.56 "LED digital tube

- Display color: red, green, blue

- External Dimension: 48×29×21 mm

- Installation aperture: 46×27 mm

- Measuring rate: ≥500mS/time

- Allowable error: allowable error 1% (±1 word)

- Operating current: less than 20mA, generally 5-15mA

- Lead length: 150mm

- Net weight: 18g

- Gross weight: 25g

Extreme operating conditions

- Minimum input voltage: +0V

- Maximum input voltage: +100V

- Operating temperature: -10 to +65

- Operating humidity: 10～80% (no condensation)

- Working air pressure: 80～106kPa

- Sunlight exposure: no direct exposure