Translated from Chinese (Simplified) to English - www.onlinedoctranslator.com

Manual download link:

Flying Eagle Card (red) manual download link:https://pan.baidu.com/s/11linkZVPNTUxXueBx9Be-w Extraction code: 4sml

Features:

lSupports up to 4-axis linkage control. The 4th axis can be set as the slave axis
lThe output pulse is 100K, the minimum error interpolation algorithm is adopted, and the machining accuracy is high.
lUSB interface, suitable for any netbook, notebook, desktop with USB interface

PC compatible computer such as computer and tablet, as long as MACH3 can run, the control card can be used.

lDriver-free design, better compatible with various software and hardware environments (support WinXP and WIN7 systems)
lSupport automatic return to origin (return to zero)
lThe driven axis is automatically leveled during homing
lSupport automatic tool setting
lSupport emergency stop input
lSupport limit switch access
lSupport spindle control (PWM mode and relay mode)
lProvides 4 digital signal inputs with optocoupler isolation
lProvide up to 12 digital signal inputs, default 4
lProvides 4 relay outputs with optocoupler isolation
lSupport handwheel interface
lAnti-interference design, imported industrial grade components, high reliability
lProvide screw terminal and 2.54MM spaced cable terminal